# GAS LEAKAGE MONITORING AND ALERTING SYSTEM

TEAM ID: PNT2022TMID51469

TEAM MEMBERS:

1. S.Vinisha - 960619104042

2. P.Sudha   - 9606191040

3. S.Abinaya - 9606191040

4. M.Thangamari - 960619104041

1.INTRODUCTION:

The usage of the gas brings great problems in the domestic as well as working places. The inflammable gas such as Liquidized petroleum gas (LPG), which is excessively used in the house and at work places. The leakage of the gas causes destructible impact to the lives and as well as to the heritage of the people. So, by keeping it in the concept of the project we have determined to develop an examining system which finds the leak of LPG gas and protects the work places by taken correct precaution at correct time. This system provides the information such as when a gas leakage is noticed, sensors of in the project are used to notice the gas leakage and immediately turns ON the buzzer for the danger indication. Buzzer is a clear indication of gas leakage. By the detection of the hazardous gas the alerting message reached to the person who has control over it from the GSM. Detection of the gas leakage is important and halting leakage is important equally. The main objective of this project is that it is extremely accurate with a least cost, this project system is best to detect gas leakage and also warn people around by buzzer beep sound and an SMS is been send to the responsible person for preparatory safety calculations.

  - Project Overview:

The presence of hazardous LPG gas leakage in a domestic, work place, also, stored gases container gas which exhibits ideal characteristic is use. For that sake, an alarm unit is used to vibrate an alarm which is buzzer. Buzzer gives an audible sign of the presence of LPG volume. The sensors are widely used to detect essence of propane, iso-butane, LPG and even smoke. The sensor has an advantage to combine a sensitivity response time. If the LPG sensor senses gas leak from work place or home, sensor output goes to active low (logic-0) condition. Arduino UNO is used in the project; low signals are overlooked by the Arduino and gas leakage is been noticed by the Arduino. The Arduino UNO turns on the LCD and buzzer. It even turns on the GSM modem after that, it continues to send messages SMS to mobile number specifically mentioned in the program of the source code for alerting danger to the people.

  - Purpose:

The purpose of Gas Leakage Monitoring and Alerting System can also be used commercially in in-house buildings to detect the presence of carbon dioxide and other combustible gases.

An IOT-enabled gas monitoring system is designed specially to prevent explosions and fire disasters in the facilities and thus save human lives.

2.LITERATURE SURVEY:

| SR.NO | Paper Title | Author Name | Publication Year | Result |
| --- | --- | --- | --- | --- |
| 1. | Internet of Things (IOT) Based Gas Leakage Monitoring and Alerting System with MQ-2 Sensor | Rohan Chandra Pandey, Manish Verma, Lumesh Kumar Sahu | 2017 | This paper choice of using a real time gas leakage monitoring and Sensing the output levels of gas has been clearly observed by the help of this system. |
| 2. | Gas Leakage Detection and Smart Alerting and Prediction Using IoT | Asmita Varma, Prabhakar S, Kayalvizhi Jayavel | 2017 | The proposed gas leakage detector is promising in the Field of safety. |
| 3. | IOT Based Gas Leakage Detection System with Database Logging, Prediction and Smart Alerting | Chaitali Bagwe, Vidya Ghadi, Vinayshri Naik, Neha Kunte | 2018 | The system provides constant monitoring and detection of gas leakage along with storage of data in database for predictions and analysis. The IOT components used helps in making the system much more cost effective in comparison with traditional Gas detector systems. |
| 4. | Internet of Things (IoT) Based Gas Leakage Monitoring and Alerting System with Mq-6 Sensor | Rohan Chandra Pandey, Manish Verma, Lumesh Kumar Sahu, Saurabh Deshmukh | 2018 | A discussion on how the aims and objectives are met is presented. An overall conclusion IOT based toxic gas detector is it has become more efficient, more applicable to today’s applications and smarter |
| 5. | Gas Leakage Detection and Smart Alerting System Using IoT | Shital Imade, Priyanka Rajmanes, Aishwarya Gavali | 2018 | In this paper we use IOT technology for enhancing the existing safety standards. While making this prototype has been to bring a revolution in the field of safety against the leakage of harmful and toxic gases |

  - Existing Problem:

A Arduino based MQ2 sensor which will detect gas leakage. We know that fire attacks due to gas leakage in buildings,restaurants,etc are increased severely. If the gas knob is left open accidently for few hours it only smells and does not catch fire, but it kept open for a long time, and then if there is any ignition then the gas rapidly catches fire.

In vehicles where we use CNG, if there is any leakage then there is also some possibility of the vehicle catching fire. The sensor in the circuit will sense the leakage, if the leakage is of high intensity it will put the alarm or the buzzer on so  the owner will be alert so as to take precaution measures to minimise the possibility to catch fire or any loss to life or property.

This is a Robust Gas Sensor suitable for sensing LPG, Smoke, Alcohol, Popane, Hydrogen, Methane and Carbon Monoxide concentrations in the air.

If you are planning on creating an indoor air quality monitoring system; breath checker or early fire detection system, MQ2 Gas Sensor Module is a great choice. MQ2 is one of the commonly used gas sensors in MQ sensor series.

  - Reference:

- Mr. Sameer Jagtap, Prajkta Bhosale, Priyanka Zanzane, Jyoti Ghogare, “LPG Gas Weight and Leakage Detection System Using IoT”,International Journal for Research in Applied Science & Engineering Technology”, Volume 4, Issue 3, March 2016, Pg – 716 to 720.

- Arun Raj, Athira Viswanathan, Athul T S, “LPGGas Monitoring System”, International Journal Innovative Technology and Research,Volume 3, Issue 2, February 2015, Pg – 1957 to 1960.

- S Shyamaladevi, V. G. Rajaramya, P. Rajasekar,P. Sebastin Ashok, “ARM7 based automatedhigh-performance system for lpg refill booking& leakage detection”, Journal of VLSI Design and Signal Processing”, Volume 3, Issue 2, 2014

- S. Sharma, V. N. Mishra, R. Dwivedi, R. Das,“Classification of gases/odours using Dynamic Response of Thick Film Gas Sensor Array”,IEEE Conference on Sensors Journal, 2013.

- Rajeev B. Ahuja, Jayant K. Dash, Prabhat Shrivastava, “A comparative analysis of liquefied petroleum gas (LPG) and kerosene related burns”, Burns, Volume 37, Issue 8, December 2011, Pg – 1403 to 1410.

- Prof. Pankaj C. Warule, Shivam Upadhyay, Snehal S. Shelke, Sumitra K. Khandade, “LPG Detection, Metering and Control System Using Microcontroller”, IJARIIE, Volume 2, Issue 2, 2016, Pg – 648 to 652.

- Ankit Sood, Babalu Sonkar, Atul Ranjan, Mr.Ameer Faisal, “Microcontroller Based LPG Gas Leakage Detector Using GSM Module”,International Journal of Electrical and Electronics Research, Volume 3, Issue2, April-June 2015, Pg – 264 to 269

- Ashish Shrivastava, Ratnesh Prabhakar, Rajeev Kumar, Rahul Verma, “GSM Based Gas Leakage Detection System”, International Journal of Technical Research and Applications”, Volume 1, Issue2, May- June 2013, Pg – 42 to 45.

- Shivalingesh B. M, Ramesh C, Mahesh S. R, Pooja R, Preethi K. Mane, Kumuda S, “LPG  Detection, Measurement and Booking System”, IJRSI, Volume 1, Issue 4, November 2014, Pg –7 to 10.

- C. Selvapriya, S. Prabha Sathya, M.Abdulrahim, C. K. Aarthi, “LPG Leakage

Monitoring and Multilevel Alerting System”, International Journal of Engineering

Sciences & Research Technology, Volume2,Issue 11, November 2013, Pg – 3287 to

3290 .

2.3 Problem Statement Definition:

Gas leakage is nothing but the leak of any gaseous molecule from a stove, or a pipeline, or cylinder etc. This can occur either purposefully or even unintendedly. As we are aware that these kinds of leaks are dangerous to our health, and when it becomes explosive it could cause great danger to the people, home, workplace, industry and the environment.

Few of the major incidents that took place due to gas leakage include the Bhopal Disaster and the Vizag Gas leak. The Bhopal disaster is known to be the worst industrial accident ever. Approximately 45 tons of Methyl Isocyanate was leaked from this insecticide plant. Methyl Isocyanate is an organic compound and a chemical that could come from the carbamate pesticides. This colorless, poisonous and flammable liquid is something that human beings have to be away from.

Vizag Gas leak was a resultant of the escape of styrene that were unattended for a long period. This colorless oily liquid can spread in fumes. So, a detector must be made in such a way that could detect any kind of gas, fume, leak, smoke etc. However harmful and dangerous it can be, the detector could be attached with certain parameters that could help to prevent the issue.

3.IDEATION AND PROPOSED SOLUTION:

3.1 Empathy Map Canvas:

3.2 Ideation and Brain Storming:

3.3 Proposed Solution:

Project team shall fill the following information in proposed solution template.

| S.No | Parameter | Description |
| --- | --- | --- |
| 1. | Problem statement | - To detect the gas leakage and stop them |
| 2. | Idea/Solution description | - Buzzer gives an audible sign of the presence of LPG volume. The sensors are widely used to detect essence of propane, iso-butane, LPG and even smoke. - Arduino UNO is used in the project; low signals are overlooked by the Arduino and gas leakage is been noticed by the Arduino. The Arduino UNO turns on the LCD and buzzer. |
| 3. | Novelty/Uniqueness | - Arduino UNO; MQ2 Gas Sensor; GSM Modem; LCD; Buzzer. |
| 4. | Social impact/Customer satisfaction | - Add great value in terms of security to industries. |
| 5. | Business model(Revenue model) | - Will be great business to the industries who prefer high security. |
| 6. | Scalability of the solution | - Will be solution and alerting workers and save the workers by alerting sensors. |

3.4 Problem Solution Fit:

4. REQUIREMENT ANALYSIS:

4.1 Functional Requirements:

Arduino: Arduino is a single-board microcontroller that is widely used to create various types of digital devices, block diagram

shown in Figure. You can control and interact with various electronics components such as sensors, actuators and much more. It

has its own fixed RAM and stores data quickly memory and EEPROM. It uses languages such as C, C ++, and Java.

Gas Sensor: Gas Sensor (MQ5) module is useful for gas leakage detection (in home and industry). It is suitable for detecting

H2, LPG, CH4, CO, Alcohol. Due to its high sensitivity and fast response time, measurements can be taken as soon as possible.

The sensitivity of the sensor can be adjusted by using the potentiometer.

Buzzer: Buzzer or Beeper is an audio signing tool, which can be mechanical, electromechanical, or piezoelectric. Buzzer and

beepers is widely used in include alarm devices, timers, and user input verification such as mouse clicks or keys

GSM Module: GSM module is a hardware device that uses telephony technology to provide a data connection in a remote network. From a mobile phone network view, they look exactly like a normal cell phone, including the need for a self-identifying SIM network. GSM modems typically provide a TTL-level serial interface to their Host. They are often used as part of an embedded system

Relay Module: The Relay module is an electrical switch used by an electromagnet. The electromagnet is opened by a low power signal that differs from a small controller. When activated, the electromagnet pulls on or off the electrical circuit

NodeMC : NodeMCU is an inexpensive open source platform. Originally included firmware running on ESP8266 Wi-Fi SoC from Express  if Systems, as well as Hardware based module ESP-12.Later, support for ESP32 32-bit MCU was added.

4.2 Non-Functional Requirements:

Non-functional requirements "refer to behavioural properties that the system must have, such as performance and usability".

5. PROJECT DESIGN:

5.1 Data Flow Diagram:

Relay

Buzzer

Microcontroller

LCD Display

If LPG gas limit

Mq2 sensor

Yes

No

GSM &

Nodecmu

End User

5.2 Solution & Technical Architecture:

5.3 User Stories:

The design of a sensor-based automatic gas leakage detector with an alert and control system has been proposed.This is an affordable, less power using, lightweight, portable, safe, user friendly, efficient, multi featured and simple system device for detecting gas. Gas leakage detection will not only provide us with significance in the health department but

it will also lead to raise our economy, because when gas leaks it not only contaminates the atmosphere, but also wastage of gases will hurt our economy. The need for ensuring safety in workplaces is expected to be the key driving force for the market over the coming years

6. PROJECT PLANNING & SCHEDULING:

6.1 Sprint Planning & Estimation:

Sprint planning is nothing but the development of codes. We use java code for simulaton.

The Sprint Planning will be splited into 4. Based on the time periods, we creating sprint1,2,3 and 4.

6.2 Sprint Delivery & Schedule:

| 1 | Identify The Problem |
| --- | --- |
| 2 | Prepare a Abstract, Problem Statement |
| 3 | List a Required Object Needed |
| 4 | Create a Code and Run it |
| 5 | Make a Prototype |
| 6 | Test with the created code and checked the design prototype is |
| 7 | Solution for the problem is found |

6.3 Reports from JIRA:

One part of ensuring the success and smooth operations of your projects in JIRA is reporting. It involves gaining the knowledge about the health, progress and overall status of your JIRA projects through Gadgets, report pages or even third party applications. The goal of this guide is to provide an overview of the tools available to JIRA users today and how they can be used to fulfill the different types of reporting needs that users face today.

7.CODING & SOLUTIONING:

7.1 Features:

Huge industries, especially oil refineries are severely exposed to harmful explosions and human risks even with a slightest of the gas leak. Moreover, the expenses due to gas leakages are enormous. This ranges from the production loss in food industries to the infrastructure losses by flammable gas leaks including the healthcare costs to treat the patients injured by gas poisoning. In the USA alone, the treatment costs linked to carbon monoxide intoxication is over $1.3 billion and the death losses due to this are over 2000 per year. Hence, advanced level gas sensors or gas monitoring systems help the manufacturers to procure accurate and real-time information regarding accidental gas leaks.

The latest versions of gas sensors are powered with IoT concepts to create live alerts in case of a gas leak or toxic atmosphere. The Internet of Things further helps the industrialists to make better decisions and analyze the situations for enhanced productivity and less infrastructure damage due to gaseous explosions.

A gas monitoring solution not only detects toxic gases but also identifies changes in air quality. It can also be used commercially in in-house buildings to detect the presence of carbon dioxide and other combustible gases. An IoT-enabled gas monitoring system is designed specially to prevent explosions and fire disasters in the facilities and thus save human lives.

8. TESTING:

8.1 Testing Cases:

several testing on the functionality of this system were conducted: (i). Microcontrollers functionality testing; (ii). SMS functionality testing; (iii). Load detector functionality testing; (iv). Gas detectors functionality testing; (v). LCD display functionality testing; (vi). LED lighting functionality testing; (vii). Alarm functionality testing. System functionality testing was conducted to ensure that the LPG Mass Monitoring Scale with Automatic Gas Leakage Detector System able to operate as designed. The Arduino is the main microcontroller to give instructions to each component in the system. The LCD screen and three LEDs with green, yellow and red colour play a role as physical displays of the cooking gas volume level. When the gas level has reached a critical level, buzzer will turn on and buzzing to warn the user that the cooking gas is about to run out and at the same time the LCD display will display the gas status "Critical" and a red LED will turn on. Table 1 is the testing results

this function of LPG Mass Monitoring Scale with Automatic Gas Leakage Detector System.

Mass LPG (kg)             Status mass         LED (Green)                yellow              Red                    Buzzer

25.58                            Ok                       ON                         OFF                OFF                     NOT BUZZING

24.10                            Ok                       ON                         OFF                OFF                      NOT BUZZING

20.13                            OK                       OFF                        ON                 OFF                      NOT BUZZING

16.73                      CRITICAL                  OFF                        ON                 OFF                         BUZZING

16.62                      CRITICAL                  OFF                        OFF                ON                          BUZZING

8.2 User Acceptance Testing:

the device was tested by using a gas to simulate flame to ensure everything was working fine and it did.

9.RESULT:

9.1 Performance Metrics:

The aims of this system design and development are to benefit:

a. Home LPG users who can make preliminary arrangements to replace old cooking gas before it runs

out and disrupts the cooking process. Even these users can avoid fires caused by cooking gas leaks.

b. With the support of ESP32, LPG consumers are alerted in the case of a gas leak at home.

c. Restaurant owners can also preserve food quality if they can identify the remaining amount of LPG

gas early and convert it before it interferes with the cooking process.

d. Customers at the restaurant do not have to wait long for meals to be ordered as a result of the restaurant

running out of gas.

f. LPG gas dealers can alleviate the strain put on them by LPG clients who need quick gas supply

10.ADVANTAGES AND DISVANTAGES:

Advantages:

- Get real-time alerts about the gaseous presence in the atmosphere

- Prevent fire hazards and explosions

- Supervise gas concentration levels

- Ensure worker’s health

- Real-time updates about leakages

- Cost-effective installation

- Data analytics for improved decisions

- Measure oxygen level accuracy

- Get immediate gas leak alerts

Disadvantages:

- Poor stability and greater environmental impact; in particular, the selectivity of each sensor is not * and the output parameters cannot be determined. Therefore, it should not be used in places where accurate measurement is required.

- Gas interference: Cross interference from other gases can compromise the performance of a gas sensor, altering the calibration curve will result in false or inaccurate readings. Component failure: Like everything else your gas detectors are prone to wear and tear

11. CONCLUSION:

This work presents the design and implementation of gas leakage detection system. Various works on gas leakages detection system was reviewed and presented. I was discovered that some of the existent research don’t takes in to considerations the cost effectiveness for the purpose of implementation of gas leakages detection at individual/domestic uses, and not easy to be further modified. This research work had advanced in knowledge as it included an embedded system to alert users via multiple mobile phones for further action to be taken when leakage is detected. The device detects gas leakage using a highly sensitive MQ-2 gas sensor to activate a buzzer that alert people of leakages, and also sent an SMS with the information “Gas Leakage Detected” from the SIM800 GSM Module as a backup to alert the appropriate authority or facility owner of a gas leakage. This design could be adopted, funded, and implemented as it has a great potential of mitigating against accidents associated with LPG leakage.

12.FUTURE SCOPE:

The further study for this system should focus on the improvement of efficiency of the product functionality. Among the suggestions are:

- Using materials that are less susceptible to fire risk for measurement scale product frames and liquefied petroleum gas (LPG) leak detectors such as stainless steel materials.

- Fix product offset problems by choosing a more stable and good load detector.

- The position of the LCD and LED displays is placed on a conspicuous part.

- Improvements to the position of gas detectors that need to be placed in the area close to the source of gas output.

- Minimize the size of the product so that the product can be placed on a gas cooking cabinet space that has a small space size.

13.APPENDIX:

Source Code:

SPRINT 1

#include <LiquidCrystal.h>

LiquidCrystal lcd(5,6,8,9,10,11);

int redled = 2;

int greenled = 3;

int buzzer = 4;

int sensor = A0;

int sensorThresh = 400;

void setup()

{

pinMode(redled, OUTPUT);

pinMode(greenled,OUTPUT);

pinMode(buzzer,OUTPUT);

pinMode(sensor,INPUT);

Serial.begin(9600);

lcd.begin(16,2);

}

void loop()

{

int analogValue = analogRead(sensor);

Serial.print(analogValue);

if(analogValue>sensorThresh)

{

digitalWrite(redled,HIGH);

digitalWrite(greenled,LOW);

tone(buzzer,1000,10000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("ALERT");

delay(1000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("EVACUATE");

delay(1000);

}

else

{

digitalWrite(greenled,HIGH);

digitalWrite(redled,LOW);

noTone(buzzer);

lcd.clear();

lcd.setCursor(0,0);

lcd.print("SAFE");

delay(1000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("ALL CLEAR");

delay(1000);

}

}

SPRINT 2

#include <LiquidCrystal.h>

LiquidCrystal lcd(5,6,8,9,10,11);

int redled = 2;

int greenled = 3;

int buzzer = 4;

int sensor = A0;

int sensorThresh = 400;

void setup()

{

pinMode(redled, OUTPUT);

pinMode(greenled,OUTPUT);

pinMode(buzzer,OUTPUT);

pinMode(sensor,INPUT);

Serial.begin(9600);

lcd.begin(16,2);

}

void loop()

{

int analogValue = analogRead(sensor);

Serial.print(analogValue);

if(analogValue>sensorThresh)

{

digitalWrite(redled,HIGH);

digitalWrite(greenled,LOW);

tone(buzzer,1000,10000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("ALERT");

delay(1000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("EVACUATE");

delay(1000);

}

else

{

digitalWrite(greenled,HIGH);

digitalWrite(redled,LOW);

noTone(buzzer);

lcd.clear();

lcd.setCursor(0,0);

lcd.print("SAFE");

delay(1000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("ALL CLEAR");

delay(1000);

}

}

SPRINT 3

#include <LiquidCrystal.h>

LiquidCrystal lcd(5,6,8,9,10,11);

int redled = 2;

int greenled = 3;

int buzzer = 4;

int sensor = A0;

int sensorThresh = 400;

void setup()

{

pinMode(redled, OUTPUT);

pinMode(greenled,OUTPUT);

pinMode(buzzer,OUTPUT);

pinMode(sensor,INPUT);

Serial.begin(9600);

lcd.begin(16,2);

}

void loop()

{

int analogValue = analogRead(sensor);

Serial.print(analogValue);

if(analogValue>sensorThresh)

{

digitalWrite(redled,HIGH);

digitalWrite(greenled,LOW);

tone(buzzer,1000,10000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("ALERT");

delay(1000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("EVACUATE");

delay(1000);

}

else

{

digitalWrite(greenled,HIGH);

digitalWrite(redled,LOW);

noTone(buzzer);

lcd.clear();

lcd.setCursor(0,0);

lcd.print("SAFE");

delay(1000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("ALL CLEAR");

delay(1000);

}

}

SPRINT 4

#include <LiquidCrystal.h>

LiquidCrystal lcd(5,6,8,9,10,11);

int redled = 2;

int greenled = 3;

int buzzer = 4;

int sensor = A0;

int sensorThresh = 400;

void setup()

{

pinMode(redled, OUTPUT);

pinMode(greenled,OUTPUT);

pinMode(buzzer,OUTPUT);

pinMode(sensor,INPUT);

Serial.begin(9600);

lcd.begin(16,2);

}

void loop()

{

int analogValue = analogRead(sensor);

Serial.print(analogValue);

if(analogValue>sensorThresh)

{

digitalWrite(redled,HIGH);

digitalWrite(greenled,LOW);

tone(buzzer,1000,10000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("ALERT");

delay(1000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("EVACUATE");

delay(1000);

}

else

{

digitalWrite(greenled,HIGH);

digitalWrite(redled,LOW);

noTone(buzzer);

lcd.clear();

lcd.setCursor(0,0);

lcd.print("SAFE");

delay(1000);

lcd.clear();

lcd.setCursor(0,1);

lcd.print("ALL CLEAR");

delay(1000);

}

}

GITHUB & PROJECT DEMO LINK:

Project Demo Link: https://drive.google.com/drive/folders/1wA4wNef1ExDWdS86iAASRQ-VyV49h0GO?usp=sharing